# Flow chart for Bodyguard T Syringe Pump Compatibilities

Always SEEK ADVICE from a PHARMACIST IF YOU ARE UNSURE or NEED GUIDANCE

Is the combination* you require:

- In the amount you require (for EACH drug)

- In the volume you require (maximum of 22ml in 30ml syringe)

- In the diluent**you require

Listed as compatible over 24 hours in:

NHS Scotland Palliative Care Guidelines

https://rightdecisions.scot.nhs.uk/scottish-palliative-care-guidelines/

YES

- Check diluent is circled on syringe pump (CSCI) prescription chart

- Note source of information in compatibility box on CSCI chart and proceed with infusion

No

YES

“The Syringe Driver” book *** (Dickman) 4th edition

(copies kept by palliative care pharmacists/technicians, hospices,

hospital medicines information centres, pharmacies in community

pharmacy palliative care network)

No

YES

Drug Compatibility Checker can be accessed online via www.medicinescomplete.com using an NHS email/Athens password***:

http://www.staffnet.ggc.scot.nhs.uk/Acute/Rehab%20Assessment/Palliative%20Care/PCRF/Documents/DRUG%20COMPATIBILITY%20CHECKER.doc

No

* Maximum number of drugs to be mixed in one syringe is 3. In exceptional circumstances, 4 may be used but whenever possible this should be discussed with a pharmacist.

Seek further advice e.g. palliative care pharmacist or hospital pharmacy medicines information service.

Consider (at any step) alternative medicines / additional pump / alternative routes or SC bolus.

** If more than one diluent is listed, normally use water. Consider sodium chloride 0.9% [only if listed] if site irritation is a problem.

*** Many of the combinations listed in these sources come from use in clinical practice and may reflect use on a single occasion; close attention to monitoring for compatibility, site problems, occlusion and infusion timing is important.

Approved by GGC Palliative Care Pharmacists           January 2011

Revised Mar 2023